1

[image: image1.png]SERENT S)50)





Updates to the Serpent 950 manual taken from the online I-Manual

	Step
	Page
	Remark

	3.1
	10
	Fastener change - 
C-clip R5 (5mm) has been replaced by the R4 C-clip 




(4mm)



	3.3
	11
	Fastener change - 
E18 (4x8mm) is wrong and has been removed




H19 (4x10mm) is wrong and has been removed




H12 (3x10mm grub screw) is used for up stop screw






	3.4
	11
	The length of connecting rod has changed from 49mm to 47.5mm. This is to avoid contact with the upper wishbone during large suspension travel.


	4.1
	14
	Note: The fuel tank is produced with a volume of 125cc. Due to manufacturing tolerances and temperature differences, we advise to have the fuel tank size checked by the organizer before participating in a major event. When the fuel tank is slightly bigger than the max. 125cc, we advise to use a heat-gun to heat the front bottom part of the fuel tank, and to gently press a dimple in the tank to reduce the volume.



	5.1
	17
	New image has been created clarifying the correct screw type for the correct mounting hole.



	5.2
	17
	New image has been created clarifying the correct screw type for the correct mounting hole.



	5.3
	18
	Fastener change - 
Screw 3x6mm (E10) - Image in the manual is not to 




scale.


	5.5
	18
	Picture change – The image now shows the placement of 2 small shims placed between the front shock tower and the X-bracket mount.



	7.2
	22
	Picture change – It now displays the black flyweight and in a different orientation to previously shown. 

Please Note. The flyweights can only be mounted one way





Latest Optional list

	[image: image2.jpg]



	#1666

Transponder holder carbon star

Introducing this essential item for all competition minded racers. This carbon fibre transponder mount comes with the option to mount both the standard transponder and the AMB personal transponder. Suitable for the Serpent 705, 835 and 950 this item mounts directly to the radio plate and is held firmly in place with two mounting screws.



	[image: image3.jpg]



	#902156
Aluminium Radio plate bracket

Beautifully machined from a single piece of aluminium this radio plate bracket is a direct replacement for the original Serpent 950 composite version. Not only does it look great it also increases the chassis stiffness due to its rigid design. An absolute must for those who like their cars looking their best.



	[image: image4.jpg]



	#902160
Pushrod set 950

Using the flexibility that the Serpent 950’s X-bracket design allows, Serpent are proud to introduce to you this front suspension pushrod conversion kit. This set comes complete with all the items required to mount the pushrod system, including non-linear progressive rockers. Due to the ease of installation the user can quickly swap suspension systems and make use of the faster steering response that comes as a result of this optional system being installed.



	[image: image5.jpg]



	#902207
Suspension brackets alu FR UP L+R
These beautifully machined front upper suspension brackets directly replace the composite versions and consequently add more strength in a heavy impact but also add the benefit of making your car look great. Available as a set, these items are essential for those drivers who want their cars with that concourse-winning look.



	[image: image6.jpg]



	#902210/11
Spacers Roll centre carbon 0.8 / 1.5
New from Serpent for use with the 950 are these carbon fibre roll centre spacers. Available in two optional sizes of 0.8mm and 1.5mm as well as the standard 3.0mm they have a large effect on the front-end grip. Increasing the amount of spacers you place on your car the higher the roll centre and the less grip that you have at the front, or alternatively the less spacers, the lower the roll centre and more the grip it produces which would result in more steering and especially on throttle.



	[image: image7.jpg]



	#909335
Anti-Roll bar rear adjustable

An essential tuning option from Serpent, beautifully machined from aluminium, this fully adjustable rear anti-roll bar features two adjustable blades that are locked in place by two grub screws, which allow for easy and quick adjustment. As per the front adjustable roll bar the rear anti-roll bar when adjusted creates a noticeable difference in handling. In the softest setting it is already harder than the standard 3.0mm anti-roll bar.



	[image: image8.jpg]



	#909513
Centax-II clutch bell housing light
Introducing this ultra-lightweight spring steel bell housing for your Centax-II clutch system. Weighing 10% less than the original bell housing this stylish looking item still retains all its strength. Due to its new design the bell housing and the rest of the Centax are now better cooled. This item is suitable for all Serpent Cars installed with the Centax-II system.




#95683 Version 1 (07/02/03)





Serpent 950 Update sheet 





On this update sheet you will find important updates to the instruction manual as well as the latest optionals and other relevant information. 





When building the car ensure that you make note of the changes in the printed manual and note them in your instruction manual to remind you to refer back to this sheet when you reach that point in the building process.





To find updated and detailed images go to www.myTSN.com/serpent950 and check the I-Manual section where you can find the latest most up to date building instructions.











    SERPENT

